README: Antonio Torres-Méndez and Sophie Bonnal Origin of Microexons

MassIVE title:

Irimia_Origin_of_Microexons_FLAG_VS21

This dataset consists of 24 raw MS files and associated peak lists and result files, acquired on a TripleTOF 6600 spectrometer (AB SCIEX, Concord, Ontario, Canada) operated in Data Dependent Acquisition mode.  cDNA constructs were supplied by Antonio Torres-Méndez (Irimia lab, Barcelona). Stable cell lines and samples were prepared by Jonathan Roth (Gingras lab, Toronto). These files are associated with a manuscript by Antonio Torres-Méndez and Sophie Bonnal et al. that describes the origin and mechanism of neuronal microexons in vertebrate species. The corresponding author on this manuscript is Dr. Manuel Irimia (mirimia@gmail.com); Dr. Anne-Claude Gingras should be contacted for questions on the AP-MS dataset (Gingras@lunenfeld.ca).

This submission is associated with 3 Supplementary Files (in addition to this README file)

Table 1 describes the composition of this dataset

Table 2 lists all the peptide identification evidence (as per iProphet)

Table 3 lists the SAINTexpress interactions

Internal reference from the Gingras lab ProHits implementation:

Project: P105

Export version: VS21

SAINT Task IDs: 3514

Experimental Procedures

Generation of stable HEK293 Flp-In T-REx Stable cell lines

Flp-InTM (Thermo Fisher Scientific) compatible expression plasmids (5’ pcDNA5-BirA*-FLAG, modified from Thermo Fisher Scientific V6010-20) containing full length and mutant human SRRM4 and SRRM3 cDNA constructs were generated using Gateway cloning (Thermo Fisher Scientific). For each bait protein, tetracycline inducible Flp-In T-Rex 293 stable cell lines were generated by transfecting 200 ng of the pcDNA5-BirA*-FLAG bait construct with 2 g of pOG44 Flp-recombinase expression plasmid (Thermo Fisher Scientific, V600520) using jetPRIME transfection reagent (Polyplus, CA89129-924). Cells containing stability integrated cDNA were selected in 200 g/mL Hygromycin B until visible colonies were formed and passaged to a 150 mm dish. Stable HEK293 Flp-In T-REx cell lines expressing BirA*-FLAG, BirA*-FLAG-NLS, or FLAG without a bait cDNA were generated in parallel to be used as negative controls.

For FLAG affinity purification experiments, all HEK293 Flp-In stable cell lines including controls were grown to 70% confluency and induced for 24 hours with 1 g/mL tetracycline to express the BirA*-FLAG tagged bait protein. After induction, each 150 mm plate was harvested by scraping in 1 mL ice-cold PBS, pelleted, and then frozen at -80C until purification.

FLAG Affinity Purification Coupled with Mass Spectrometry (AP-MS) Sample Preparation

FLAG AP-MS experiments were performed in two biological replicates essentially as described in Lambert et al. 2015 (PMID: 25281560) with slight modifications. Roughly, cell pellets for each bait protein were lysed in ice cold TAP lysis buffer containing 50 mM HEPES-NaOH (pH 8.0), 100 mM NaCl, 2 mM EDTA, and 10% glycerol with freshly added 0.1% NP-40, 1 mM DTT, 1 mM PMSF, and Protease inhibitor cocktail (Sigma-Aldrich P8340, 1:500) at a 1:4 pellet weight to volume ratio (for 0.1g add 0.4 mL of lysis buffer).

Resuspended pellets were frozen and thawed by placing the pellets on dry ice for 5 minutes and thawed in a 37 C water bath until only a small ice pellet remained. Samples were quickly moved back on to ice and sonicated with three 10 second bursts with 2 second rest at an amplitude of 35%. To solubilize chromatin and reduce the detection of interactions mediated by RNA or DNA, 1 L of Benzonase Nuclease (Sigma-Aldrich, E8263, 250U) was added to each tube and incubated 30 minutes with rotation at 4C.

Lysates were cleared by centrifugation at 20,000 rcf for 20 minutes at 4C and the lysates transferred to tubes containing 25 L of 50% magnetic anti-FLAG M2 beads slurry (Sigma-Aldrich, M8823) pre-washed in lysis buffer. FLAG immunoprecipitation was allowed to proceed for 3 hours at 4C with rotation.

After incubation, beads were pelleted by centrifugation (1,000 rpm for 1 minute) and magnetized to aspirate the unbound lysate. The beads were then demagnetized and washed with 1 mL of lysis buffer and the total volume (with beads) transferred to a new tube. Beads were washed once more with 1 mL of lysis buffer followed by one wash with 50 mM ammonium bicarbonate (ABC) at pH 8. All wash steps were performed on ice using cold lysis buffer and ABC. After the final wash, any residual ABC was aspirated from the beads, and 1 g of trypsin (Sigma-Aldrich, T6567) in 10 L of ABC was added to each tube. The samples were incubated at 37C overnight with rotation.

The following morning, beads were magnetized, and the supernatant transferred to a new tube. Another 250 ng of trypsin was added to each supernatant in 5 L of ABC (total volume of 15 L) and further digested with rotation for another 4 hours at 37C. Samples were acidified with formic acid to a final concentration of 2.5% and dried in a centrifugal evaporator.

AP-MS Data Acquisition

Digested peptides were dissolved in 5% formic acid in a volume in which 6 μL contained purified material from one-quarter of a 150 mm plate. For each sample, 5 L of each sample was directly loaded at 800 nL/min onto a 15 cm 100 m ID emitter tip packed in-house with 3.5 m Reprosil C18 (Dr. Maische). The peptides were eluted from the column at 400 nL/min over a 90 min gradient generated by a 425 NanoLC (Eksigent, Redwood, CA) and analyzed on a TripleTOFTM 6600 instrument (AB SCIEX, Concord, Ontario, Canada). The gradient started at 2% acetonitrile with 0.1% formic acid and increased to 35% acetonitrile over 90 minutes followed by 15 minutes at 80% acetonitrile, and then 15 min at 2% acetonitrile for a total of 120 minutes. To minimize carryover between each sample, the analytical column was flushed for 1 hour at 1500 nl/min with an alternating sawtooth gradient from 35% acetonitrile to 80% acetonitrile, holding each gradient concentration for 5 minutes. Analytical column and instrument performance were verified after each sample by analyzing 30 fmol bovine serum albumin (BSA) tryptic peptide digest with 60 fmol α-casein tryptic digest with a short 30 minute gradient. MS mass calibration was performed on BSA reference ions between each sample. Acquisition was in Data Dependent mode and consisted of one 250 millisecond MS1 TOF survey scan from 400-1250 Da followed by twenty 100 millisecond MS2 candidate ion scans from 100–2000 Da in high sensitivity mode. Only ions with a charge of 2+ to 4+ which exceeded a threshold of 200 counts per second were selected for fragmentation, and former precursors were excluded for 10 seconds after 1 occurrence.

AP-MS Data Analysis

Mass spectrometry data was stored, searched and analyzed using the ProHits laboratory information management system (LIMS) platform (PMID: 27132685). The WIFF data files were converted to MGF format using WIFF2MGF and subsequently converted to an mzML format using ProteoWizard (3.0.4468) and the AB SCIEX MS Data Converter (V1.3 beta). The mzML files were searched using Mascot (v2.3.02) and Comet (2014.02 rev.2). The results from each search engine were jointly analyzed through the Trans-Proteomic Pipeline (TPP) (PMID: 20101611) via the iProphet pipeline (PMID: 21876204). The spectra were searched against a total of 72,482 proteins consisting of the NCBI human RefSeq database (v57, Aug 9th, 2016, forward and reverse sequences) supplemented with “common contaminants” from the Max Planck Institute (http://141.61.102.106:8080/share.cgi?ssid=0f2gfuB) and the Global Proteome Machine (http://www.thegpm.org/crap/index.html) as well as sequences from common fusion proteins and epitope tags. The database parameters were set to search for tryptic cleavages, allowing up to 2 missed cleavage sites per peptide, MS1 mass tolerance of 40 ppm with charges of 2+ to 4+ and an MS2 mass tolerance of ± 0.15 amu. Asparagine/glutamine deamidation and methionine oxidation were selected as variable modifications. A minimum iProphet probability of 0.95 was required for protein identification with a minimum number of 2 unique peptides required for protein interaction scoring.

Significance Analysis of INTeractome (SAINTexpress version 3.6.1 (PMID: 24513533) was used as a statistical tool to calculate the probability value of each potential interaction from background contaminants. Briefly, our experimental design included specific negative controls (BirA*-FLAG, BirA*-FLAG-NLS, and FLAG), each run in several biological replicates (12 total). Each biological replicate of a bait was analyzed independently against these compressed controls before averaging of the score values and assessment of the Bayesian False Discovery Rates (FDR; PMID: 24513533). High-confidence interactions are those with FDR ≤ 1%.