Metabolite Extraction from Plasma/Serum/Culture Supernatant

Samples were incubated at -80 oC for at least one hour, or up to overnight. Metabolites were extracted with the addition of 4 volumes of extraction solvent (100% methanol spiked with internal standards and stored at -80 oC, see table) to one volume of liquid sample in a microcentrifuge tube. Tubes were then centrifuged at -10 oC, 20,000 x g for 15 min and supernatant was used for subsequent metabolomic analysis.

Extraction Solvent recipe: 100% Methanol with the following internal standards added

| Panel | ITSD | Vendor | Product Number | Concentration | Mass/Volume for 1L |
| --- | --- | --- | --- | --- | --- |
| SCFA | D3-Acetate | CIL | DLM-3126-25 | 4 mM | 340 mg |
| SCFA | D5-Propionate | CIL | DLM-1601-1 | 1 mM | 101.1 mg |
| SCFA | D7-Butyrate | CIL | DLM-7616-PK | .5 mM | 58.56 mg |
| SCFA | D9-Valerate | CIL | DLM-572-5 | .25 mM | 29.60 µL |
| SCFA | D8-Valine | CIL | DLM-488-0.25 | .05 mM | 6.26 mg |
| SCFA | D6-Succinate | CIL | DLM-831-PK | 1 mM | 124.14 mg |
| SCFA | D6-Phenol | CIL | DLM-370-PK | .025 mM | 2.5 mg |
| TMS | U13C-Palmitate | CIL | CLM-409-PK | .04 mM | 10.9 mg |
| TMS/SCFA | D7N15-Proline | CIL | DLNM-7562-0.25 | .02 mM | 2.46 mg |
| Bile Acids | D4-Cholic Acid | CIL | DLM-2611-0.05 | 1 ug/mL | 1 mL of 1 mg/mL stock |
| Bile Acids | D4-Deoxycholic Acid | CIL | DLM-2824-0.01 | 1 ug/mL | 1 mL of 1 mg/mL stock |
| Bile Acids | D4-Glycocholic Acid | CIL | DLM-2742-0.01 | 1 ug/mL | 1 mL of 1 mg/mL stock |
| Bile Acids | D4-Glycodeoxycholic Acid | CIL | DLM-9554-0.01 | .5 ug/mL | .5 mL of 1 mg/mL stock |
| Bile Acids | D4-Litchocholic Acid | CIL | DLM-9560-0.05 | 5 ug/mL | 5 mL of 1 mg/mL stock |
| Bile Acids | D4-Taurocholic Acid | CIL | DLM-9572-0.01 | 1 ug/mL | 1 mL of 1 mg/mL stock |
| Bile Acids | D4-Taurodeoxycholic Acid | CIL | DLM-9568-0.01 | .5 ug/mL | .5 mL of 1 mg/mL stock |
| Bile Acids | D5-Alpha-Muricholic Acid | CIL | DLM-10627-PK | 3.5 ug/mL | 3.5 mL of 1 mg/mL stock |
| Indoles | 13C11-L-Tryptophan | CIL | CLM-4290-H | 500 nM | 50 µL of 10 mM stock |
| Indoles | 13C9-L-Phenylalanine | CIL | CLM-2250-H | 500 nM | 50 µL of 10 mM stock |
| Indoles | 13C9-L-Tyrosine | CIL | CLM-2263-H | 500 nM | 50 µL of 10 mM stock |
| Indoles | D4-Serotonin | CIL | DLM-11030-0 | 500 nM | 50 µL of 10 mM stock |
| Indoles | 13C6-5-Hydroxyindole-3-acetic acid | CIL | CLM-9936-0 | 500 nM | 50 µL of 10 mM stock |
| Indoles | D4-Tryptamine | CIL | DLM-6989-0 | 500 nM | 50 µL of 10 mM stock |
| Indoles | D3-Melatonin | CIL | DLM-7101-0 | 500 nM | 50 µL of 10 mM stock |
| Indoles | 13C6-Niacin | CIL | CLM-9954-0 | 2 uM | 200 µLof 10 mM stock |
| Indoles | 13C10-L-Kynurenine | CIL | CLM-9884-0 | 500 nM | 50 µL of 10 mM stock |
| Indoles | D5-Kynurenic Acid | CIL | DLM-7374-0 | 500 nM | 50 µL of 10 mM stock |
| Indoles | Anthranilic Acid, Ring-13C6 | CIL | CLM-701-0 | 500 nM | 50 µL of 10 mM stock |

CIL = Cambridge Isotope Laboratories

Metabolite Analysis using GC-EI-MS and Methoxyamine and TMS Derivatization

Metabolites were analyzed using GCMS with Electron Impact Ionization. 100uL of metabolite extract was added to prelabeled mass spec autosampler vials (Microliter; 09-1200) and dried down completely under nitrogen stream at 30 L/min (top) 1 L/min (bottom) at 30 oC (Biotage SPE Dry 96 Dual; 3579M). To dried samples, 50 µL of freshly prepared 20 mg/mL methoxyamine (Sigma; 226904) in pyridine (Sigma; 270970) was added and incubated in a thermomixer C (Eppendorf) for 90 min at 30 °C and 1400 rpm. After samples are cooled to room temperature, 80 µL of derivatizing reagent (BSTFA + 1% TMCS; Sigma; B-023) and 70 µL of Ethyl Acetate (Sigma; 439169) were added and samples were incubated in a thermomixer at 70 oC for 1 hour and 1400rpm.  Samples were cooled to RT and 400 µL of Ethyl Acetate was added to dilute samples. Turbid samples were transferred to microcentrifuge tubes and centrifuged at 4 oC, 20,000 x g for 15 min. Supernatants were then added to mass spec vials for GCMS analysis. Samples were analyzed using a GC-MS (Agilent 7890A GC system, Agilent 5975C MS detector) operating in electron impact ionization mode, using a HP-5MSUI column (30 m x 0.25 mm, 0.25 µm; Agilent Technologies 19091S-433UI) and 1 µL injection. Oven ramp parameters: 1 min hold at 60 oC, 16 oC per min up to 300 oC with a 7 min hold at 300 oC. Inlet temperature was 280 oC and transfer line was 300 oC. Data analysis was performed using MassHunter Quantitative Analysis software (version B.10, Agilent Technologies) and confirmed by comparison to authentic standards. Normalized peak areas were calculated by dividing raw peak areas of targeted analytes by averaged raw peak areas of internal standards.